Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    September 4, 2018

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          September 19, 2018

PROJECT:      1010 Zhingob Ave., Hinckley MN

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a residential renovation at 1010 Zhingob Avenue, Hinckley MN.   Bids will be due Wednesday September 19, 2018 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday September 20, 2018 at 8:30 AM.

A mandatory Pre-Bid site visit will be held on Friday, September 7, 2018 at 3:00 p.m. or immediately following the Pre-Bid site visit at 2372 Wiigwaas St., Hinckley.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:

Exterior:

- Demo and replace all damaged siding and accessories, including shutters on front side of home. All replaced siding shall be color matched to existing.

- Contractor shall replace the entire top row of siding on the rear of the home and the j-channel too, if needed.

- Pressure wash and clean all siding and exterior surfaces.

- Demo and replace exterior light fixtures with spec product. Supply and install all new LED bulbs.

- Demo and replace overhead garage door, weatherstripping and metal jamb trim for overhead garage door, match existing color.

- Supply and install a 4”x4” post for the front stoop. Wrap post with either vinyl or trim board, color shall be white.

- Cap end of gutter that runs along the left side of the garage and remove existing downspout.

- Supply and install a gutter over the front stoop to be connected to the existing gutter along the garage side. Supply and install a downspout to be mounted on the newly installed stoop post. Supply and install spec splash pad.

- Demo and replace front entry door, frame and weatherstripping. Finish per Spec Book.

- Demo board beneath the front entry door sill. Supply and install pvc trim board to replace, shall be white in color.

- Demo and replace lock set on the side garage service door; and supply and install new lock sets on new front, rear and garage service entry doors with spec product. All lock sets to be keyed alike.

- Demo patio door. Modify the opening to fit 3’ entry door per Spec Book. Supply and install spec entry door with storm door. Install lock set per Spec Book; to be keyed alike all other new lock sets. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

- Supply and install spec storm doors to front and back entries.

- Demo and replace all decking, handrails and stair treads for rear deck.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing or demo and replace damaged window screens.

- Replace missing or damaged gutter downspouts.

- Dispose of any existing splash pads. Contractor shall supply and install all new splashpads per the Spec Book.

- Repair soffit box end at the front left corner of the garage.

- Demo the existing garage apron and front sidewalk. Prep area with new clean sand level, compact base. Pour all new concrete to replace sidewalk and apron with #4 re-bar reinforcement @ 24” o.c. through and expansion joint between stoop and sidewalk and between apron and garage floor. Sidewalk & garage apron height shall be poured to be within ¼” of the garage floor height.

- Contractor shall supply and install 911 address sign with post to be installed by the curb.

- Contractor shall reattach cable box cover.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Install sheet rock for modified rear entry door opening. Texture, paint and finish, including installation of baseboard stained to match existing finish.

- Supply and install new garage service door and frame. Install new lock set to be keyed alike front and rear entries.

- Supply and install new cover for attic access located in the garage.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing  as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint Dover White. For the lower level, demo and replace any sections of drywall with heavy molding, including replacement of all bottom 24” of drywall throughout the lower level.

- Demo and replace; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace; or supply and install any damaged or missing floor vents, grilles, diffusers and return air covers with spec products.

- Remove existing ceiling fan. Supply and install new surface mount light fixtures and controllers.

- Supply and install all interior light globes with Spec product. Demo and replace any damaged interior light fixtures with Spec product, to include all new light bulbs per Spec Book schedule. New bulbs shall all be LED.

- Demo and replace flooring throughout upper floor, front entry and 2 lower level bedrooms with spec sheet vinyl. To include supply and install spec underlayment. Vinyl selection to be 58021 Sunstone. Grind and prep subfloors as needed for a neat even finished appearance.

- Demo bi-fold closet doors on front entry closet. Supply and install double doors per Spec Book, stain and hardware to match existing finishes.

- Replace wrinkled vinyl stair treads, match to existing.

- Demo and replace range hood, spec unit vented to the exterior.

- Supply and install new range and refrigerator per Spec Book.

- Demo and replace kitchen faucet with Spec product.

- Demo and replace all lower kitchen cabinets and island cabinet, match to existing finishes. Supply and install missing top trim for pantry cabinet.

- Remove and dispose of existing dishwasher. Supply and install 24” lower cabinet for opening where dishwasher was located. All new hardware and stain should match existing finishes.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install missing shelving or hardware. All new hardware to match existing finishes. Adjust all cabinets.

- Demo and replace all millwork throughout the home, including stair rails, balusters and other components. Match to existing finishes.

- Demo and replace base-boards throughout with spec products. Base stain to match existing finishes in home. Spec vinyl base to be installed in bathrooms and mechanical room.

- Demo and replace window extension jambs. To include spray foaming during installation.

- Demo and replace window casings with spec products, matching existing finishes.

- Thoroughly clean all window surfaces.

- Demo and replace all floor vents, diffusers and return air covers with spec products.

- Demo and replace bath vanities with spec products reusing the existing faucets and sinks. Thoroughly clean sinks.

- Demo and replace; or supply and install all new bathroom accessories per Spec Book.

- Demo and replace showerheads with Moen product similar to Spec model.

- Demo and replace bath fans with spec products vented to the exterior.

- Thoroughly clean tubs and surrounds.

- Demo and replace toilet stools in both the upper and lower bathrooms.

- Re-caulk counter tops, toilet stool bases, tub/shower surrounds, as needed.

- Supply and install door-stops throughout with spec product.

- Demo all existing closet shelving. Supply and install wire closet shelving per Spec Book throughout the home.

- Demo and replace all interior doors and hardware with spec products, doors stained to match existing finishes, hardware to match existing finish. Adjust all.

- Remove casing, bi-fold doors and hardware from bedroom closets. Wrap baseboard around closet opening. Patch and finish drywall as needed for sheet rock opening. No doors to be installed.

- Demo and replace all cap board on basement half walls; stain to match existing finishes.

- Service existing furnace and HRV unit.

- Clean and paint mechanical/ laundry room floor per Spec Book.

- Insulate exposed plumbing pipes in mechanical room per Spec Book.

- Provide and install window blinds throughout with Spec product. Dispose of any existing.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact either Carla Dunkley at 320-532-7429 or by email at carla.dunkley@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Craig Hansen at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 1010 Zhingob Ave.

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

- Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@mlcv.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.  Site will also require city permitting.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2018 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  1010 Zhingob Ave., Hinckley MN

LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1: (Add to Replace Window Blinds)

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1: (Add to Replace Capboard Instead of Refinish

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________