Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    August 14, 2019

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          September 11, 2019

PROJECT:      Evergreen Court Quad Rehabs

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for three residential renovations located at 2481, 2485 & 2489 Evergreen Ct, Hinckley.   Bids will be due Wednesday September 11, 2019 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday September 12, 2019 at 8:30 AM.

A mandatory Pre-Bid site visit will be held on Tuesday, August 27, 2019 at 9:30 a.m. Starting at 2481 Evergreen Ct Unit A.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:

2481 Evergreen Ct Quad Exterior: The following allows to the entire exterior of the quad. Work specific to the exterior of each unit is listed separately.

Units A, B, C & D:

- Demo and replace all damaged siding and accessories. All replaced siding shall be color matched to existing. This shall include any siding with excessive screw holes.

- Pressure wash and clean all siding and exterior surfaces.

- Demo and replace; or supply and install any damaged or missing foundation flashing.

- Contractor shall demo and dispose of patio fence.

- Demo and replace exterior light fixtures with Spec model.

- Supply and install all new LED bulbs for exterior light fixtures.

- Demo and replace damaged or missing downspouts; and secure all.

- Cleanout all gutters.

- Supply and install spec splash pads.

- All entry door lock sets to be keyed alike, including front, patio and garage service.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing or demo and replace damaged window or patio door screens.

- Contractor shall remove and dispose of the existing dish installed on the roof. Contractor shall caulk screw holes or repair any damage to the roof.

- Contractor shall supply and install any missing outlet box covers.

- Contractor shall remove all incidental small trees and vegetation growing next to the exterior of the unit.

- Contractor shall remove and dispose of any miscellaneous yard debris.

2481 Evergreen Ct Unit A

Exterior:

- Demo and replace front storm door with chain kit.

- Demo and replace front entry door slab. New door shall be painted green to match the overhead garage door. Finish per Spec Book. Demo and replace weatherstripping. Install new lock set to be keyed alike rear entry and garage service door.

- Contractor shall thoroughly clean and scrub the overhead garage door.

2481 Evergreen Ct Unit B

Exterior:

- Repair of the siding on the right front corner of the garage.

- Demo and replace front storm door with Spec model, to include chain kit.

- Demo and replace the front entry door slab and weatherstripping. Paint new door green similar to the overhead garage and finish per Spec Book.

- Contractor shall demo and replace the overhead garage door weatherstripping and metal jambs.

- Contractor shall secure loose fascia trim on the front side of the upper floor.

- Contractor shall fill in holes in the backyard, including those next to the patio slab.

2481 Evergreen Ct Unit C

Exterior:

- Demo and replace front storm door with Spec model, to include chain kit.

- Contractor shall clean and paint front entry door green to match overhead garage door color. Finish per Spec Book.

2481 Evergreen Ct Unit D

Exterior:

- Adjust and clean front storm door.

- Adjust, clean and repaint front entry door a green similar to the overhead garage door color. Finish per Spec Book. Demo and replace weatherstripping. Install new lock set to be keyed alike rear entry and garage service door.

2481 Evergreen Ct Unit A

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Demo and replace garage service door slab. New door shall be painted to match the existing. Finish per Spec Book. Demo and replace weatherstripping. Install new lock set with deadbolt to be keyed alike front and rear entries.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing  as closely as possible. Repaint Dover White. Prime all drywall in the bathroom with Zinsser mold killing primer. Demo and replace sheetrock in areas with extensive mold.

- Demo and replace; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace damaged; or supply and install any missing light fixtures or lighting components per Spec Book schedule

- Install new LED bulbs throughout the interior.

- Demo and replace vinyl flooring in front entry, kitchen, dining and living room area with spec sheet vinyl. Vinyl selection to be 58021 Sunstone. Clean and paint mechanical/ laundry room floor per Spec Book.

- Demo all lower kitchen cabinets and replace with same ADA cabinets. Match existing finish. Thoroughly clean and reinstall existing countertop, sink and facucet.

- Thoroughly clean all kitchen cabinet surfaces. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace existing range hood with Whirlpool model UXT4030ADW.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Thoroughly clean all window surfaces.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Supply and install bath vanity cabinet with spec products reusing the existing top, faucet and sink. Thoroughly clean sink, faucet and countertop.

- Demo existing shower stall and surround. Supply and install WR 530 FF shower with grab bars.

- Demo and replace showerhead and trim set with Moen Spec products.

- Demo and replace Broan bath fan grille and fan assembly to include motor.

- Contractor shall supply and install grab bars for toilet.

- Thoroughly clean toilet stool.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Supply and install any missing door-stops throughout with spec product.

- Adjust all interior doors.

- Service existing furnace and HRV unit.

- Thoroughly clean all interior surfaces.

2481 Evergreen Ct Unit B

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Clean, paint and adjust garage service door. Finish per Spec Book. Demo and replace weatherstripping. Install new lock set with dead bolt to be keyed alike front and rear entries.

- Contractor shall have the low voltage box inspected for damage and make any necessary repairs. Contractor shall supply and install a new cover.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing as closely as possible. Repaint Dover White.

- Demo and replace; or supply and install any missing, damaged or painted receptacle, wall plates and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo all existing flooring. Supply and install vinyl flooring throughout to include spec underlayment. Vinyl selection to be 58021 Sunstone. Clean and paint mechanical/ laundry room floor per Spec Book.

- Secure and/ or adjust vinyl stair treads to fit correctly.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace; or supply and install any missing, damaged interior light globes or trim. All per Spec Book lighting schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace existing range hood with Whirlpool model UXT4030ADW.

- Demo and replace any damaged kitchen cabinet fronts and hardware. Contractor shall demo and replace the existing toe kick. Match existing finishes.

- Demo existing dishwasher. Supply and install 24” cabinet in its place. Match existing finishes.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install liner for cabinet below sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Secure existing handrail for stairs.

- Contractor shall replace damaged or missing stair handrail components.

- Thoroughly clean all window surfaces.

- Supply and install missing; or demo and replace damaged window and patio door blinds.

- Supply and install new bath vanities in both bathrooms. Thoroughly clean cabinet, sink, faucet and countertop.

- Demo and replace showerheads with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor in each bathroom.

- Thoroughly clean tub and surround.

- Thoroughly clean toilet stools.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Supply and install any missing door-stops throughout with spec product.

- Contractor shall demo and replace 2 interior doors for upper front bedroom and bath.

- Adjust all interior doors.

- Service existing furnace and HRV unit.

- Contractor shall supply and install any missing drainage hoses for equipment in mechanical room.

- Thoroughly clean all interior surfaces.

2481 Evergreen Ct Unit C

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Clean and adjust garage service door. Finish per Spec Book. Install new lock set with dead bolt to be keyed alike front and rear entries.

- Contractor shall have the low voltage box inspected for damage and make any necessary repairs. Contractor shall supply and install a new cover.

- Contract shall demo and replace sill cock in garage.

- Contract shall demo and replace moldy sheet rock in the garage. Tape new sheetrock on firewall.

- Contract shall supply and install sheetrock to cover hole cut in garage ceiling. Contract shall supply and install insulation and poly as needed before patching and taping new sheetrock.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing as closely as possible. Repaint Dover White.

- Demo and replace; or supply and install any missing, damaged or painted receptacle, wall plates and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo all existing flooring. Supply and install vinyl flooring throughout. Vinyl selection to be 58021 Sunstone. Clean and paint mechanical/ laundry room floor per Spec Book.

- Supply and install vinyl stair treads.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace; or supply and install any missing, damaged interior light globes or trim. All per Spec Book lighting schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace existing range hood with Whirlpool model UXT4030ADW.

- Demo and replace all lower kitchen cabinet drawer and door fronts and hardware. Match existing finishes.

- Demo existing dishwasher. Supply and install 24” cabinet in its place. Match existing finishes.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install liner for cabinet below sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Secure existing handrail for stairs.

- Thoroughly clean all window surfaces.

- Supply and install missing; or demo and replace damaged window and patio door blinds.

- Supply and install any missing; or demo and replace any damaged knobs or hardware for both bath vanities. Thoroughly clean cabinet, sink, faucet and countertop.

- Demo and replace both showerheads with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor in each bathroom.

- Thoroughly clean tub and surround.

- Contractor shall demo and replace showerhead and trim set for the upper bathroom per Spec Book.

- Thoroughly clean toilet stools.

- Thoroughly clean both bath vanities, counters, sinks and faucets.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Supply and install any missing door-stops throughout with spec product.

- Adjust all interior doors.

- Service existing furnace and HRV unit.

- Thoroughly clean all interior surfaces.

2481 Evergreen Ct Unit D

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Adjust and repaint garage service door. Finish per Spec Book. Demo and replace weatherstripping. Install new lock set with deadbolt to be keyed alike front and rear entries.

- Contractor shall supply and install sheetrock and tape the right wall of the garage.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing  as closely as possible. Repaint Dover White. Note: This unit has very little drywall repair.

- Demo and replace; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Install new LED bulbs throughout the interior.

- Demo and replace all carpet flooring with spec sheet vinyl. Vinyl selection to be 58021 Sunstone.

- Demo flooring in the mechanical room. Clean and paint mechanical/ laundry room floor per Spec Book.

- Demo and replace toe kick on kitchen cabinets. Match existing finish.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install liner for cabinet beneath kitchen sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Contractor shall supply and install missing; or demo and replace damaged stair railing components. All new materials shall be stained to match existing finishes. Contractor shall reconstruct said stair railing.

- Thoroughly clean all window surfaces.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Demo and replace bath vanity cabinet only with spec products reusing the existing top, faucets and sink. Thoroughly clean sink, faucet and countertop.

- Demo and replace both showerheads with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor.

- Thoroughly clean tub and surround.

- Thoroughly clean toilet stool.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Supply and install any missing door-stops throughout with spec product.

- Adjust all interior doors.

- Service existing furnace and HRV unit.

- Thoroughly clean all interior surfaces.

2485 Evergreen Ct Quad Exterior: The following allows to the entire exterior of the quad. Work specific to the exterior of each unit is listed separately.

Units A, B, C & D:

- Demo and replace all damaged siding and accessories. All replaced siding shall be color matched to existing. This shall include any siding with excessive screw holes.

- Pressure wash and clean all siding and exterior surfaces.

- Demo and replace; or supply and install any damaged or missing foundation flashing.

- Contractor shall demo and dispose of patio fence.

- Demo and replace exterior light fixtures with Spec model.

- Supply and install all new LED bulbs for exterior light fixtures.

- Demo and replace damaged or missing downspouts; and secure all.

- Cleanout all gutters.

- Supply and install spec splash pads.

- All entry door lock sets to be keyed alike, including front, patio and garage service.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing or demo and replace damaged window or patio door screens.

- Contractor shall remove and dispose of the existing dish installed on the roof. Contractor shall caulk screw holes or repair any damage to the roof.

- Contractor shall supply and install any missing outlet box covers.

- Contractor shall remove all incidental small trees and vegetation growing next to the exterior of the unit.

- Contractor shall remove and dispose of any miscellaneous yard debris.

2485 Evergreen Ct Unit A

Exterior:

- Thoroughly clean and adjust front storm door.

- Demo and replace front entry door slab, lock jamb and weather stripping. New door shall be painted Oxford Blue. Finish per Spec Book.

- Demo and replace patio doors with similar style doors as existing.

- Contractor shall include in their base bid, the cost to demo and replace 2 bedroom windows. If windows can be repaired, contractor shall supply Owner with credit for the difference in cost.

2485 Evergreen Ct Unit B

Exterior:

- Contractor shall demo and replace cracked section of sidewalk next to the front stoop. Contractor shall supply and install any clean fill necessary to prep the area.

- Contractor shall clean and paint existing front entry door Oxford Blue. Finish per Spec Book.

- Demo and replace garage overhead door weatherstripping.

- Contractor shall carefully remove fungus growing on rear shingles. Contractor shall take great care to not damage the shingles.

2485 Evergreen Ct Unit C

Exterior:

- Contractor shall demo and replace cracked section of sidewalk next to the front stoop. Contractor shall supply and install any clean fill necessary to prep the area.

- Demo and replace damaged gutter over front stoop.

- Supply and install new lock for the overhead garage door.

- Demo and replace front storm door with Spec model, to include chain kit.

- Contractor shall clean and paint front entry door Oxford Blue. Finish per Spec book.

- Demo and replace doorbell.

- Contractor shall supply and install all materials necessary to patch the damaged areas of upper roof on the front side of the unit.

2485 Evergreen Ct Unit D

Exterior:

- Contractor shall demo and replace section of sidewalk next to the front stoop. Contractor shall supply and install any clean fill necessary to prep the area. The new section shall have a slight incline to match the elevation of the existing stoop and remaining sidewalk.

- Demo and replace front storm door with Spec model, to include chain kit.

- Demo and replace front entry door slab, lock jamb, weather stripping and hardware. Contractor shall paint new door Oxford Blue. Finish per Spec book.

2485 Evergreen Ct Unit A

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Clean and paint garage door slab for reuse. Clean and paint door frame. Finish per Spec Book. Install new lock set and dead bolt to be keyed alike front and rear entries.

- Contractor shall have the low voltage box inspected for damage and make any necessary repairs. Contractor shall supply and install a new cover.

- Contractor shall patch and tape sheetrock to repair hole in garage wall.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- For drywall surfaces that need repair or have not already been repainted: patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing as closely as possible. Repaint Dover White. Demo and replace sheetrock for areas with extensive mold.

- Demo and replace; or supply and install any missing, damaged or painted receptacle, wall plates and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace all interior light fixtures per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace all flooring with spec sheet vinyl. Vinyl selection to be 58020 Oasis Beige.

- Demo and replace all kitchen cabinets, countertop, sink and faucet to match existing finishes. This should include installing a 24” cabinet in place of dishwasher opening.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install all new baseboards, trim or casing. Match to existing finishes.

- Demo and replace; or supply and install all new interior doors.

- Contractor shall remove and dispose of sliding track hardware on bedroom closets. No doors will be installed. Patch, paint and finish sheetrock opening to match the existing finish.

- Thoroughly clean all window surfaces.

- Demo and replace; or supply and install all new window blinds throughout.

- Supply and install any missing; or demo and replace any damaged knobs or hardware for bath vanity. Thoroughly clean cabinet, sink, faucet and countertop.

- Demo and replace showerhead and trim set with Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor.

- Thoroughly clean tub and surround.

- Contractor shall demo and replace toilet.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Contractor shall demo and replace bath vanity, sink and faucet with Spec products and match existing finishes.

- Supply and install missing; or demo and replace damaged bath accessories.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Supply and install any missing door-stops throughout with spec product.

- Adjust all interior doors.

- Contractor shall supply and install new water heater per Spec Book.

- Contractor shall supply and install new standing laundry tub Spec model.

- Service existing furnace and HRV unit.

- Thoroughly clean all interior surfaces.

2485 Evergreen Ct Unit B

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Clean, adjust and paint garage service door. Finish per Spec Book. Install new lock set with dead bolt to be keyed alike front and rear entries.

- Contractor shall have the low voltage box inspected for damage and make any necessary repairs. Contractor shall supply and install a new cover.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing as closely as possible. Repaint Dover White.

- Demo and replace; or supply and install any missing, damaged or painted receptacle, wall plates and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace; or supply and install any missing, damaged interior light globes or trim.

- Install new LED bulbs throughout the interior.

- Demo and replace all flooring with spec sheet vinyl. To include supply and install spec underlayment. Vinyl selection to be 58021 Sunstone.

- Contractor shall supply and install vinyl stair treads.

- Demo and replace all lower kitchen cabinets. Match existing finish. Contractor shall include 24” cabinet where dishwasher once was.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace existing range hood with Whirlpool model UXT4030ADW.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Thoroughly clean all window surfaces.

- Contractor shall demo and replace all window and patio door blinds.

- Contractor shall demo and replace faucet in the lower bathroom.

- Supply and install any missing; or demo and replace any damaged knobs or hardware for both bath vanities. Thoroughly clean cabinet, sink, faucet and countertop in each.

- Demo and replace Broan bath fan grille and fan assembly to include motor in each bathroom.

- Thoroughly clean tub and surround.

- Contractor shall demo and replace the showerhead and trim set for the upper bathroom per Spec Book.

- Thoroughly clean toilet stools.

- Re-caulk counter tops, toilet stool bases, tub/shower surround, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Supply and install any missing door-stops throughout with spec product.

- Adjust all interior doors.

- Service existing furnace and HRV unit.

- Supply and install new hot water heater per Spec Book.

- Supply and install standing laundry tub per Spec Book.

- Thoroughly clean all interior surfaces.

2485 Evergreen Ct Unit C

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Demo and replace garage service door slab, weatherstripping and hardware. Finish per Spec Book. Install new lock set with dead bolt to be keyed alike front and rear entries.

- Contractor shall have the low voltage box inspected for damage and make any necessary repairs. Contractor shall supply and install a new cover.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all damaged interior drywall surfaces only. Texturing to match existing as closely as possible. Repaint Dover White. Note: This unit was recently repainted.

- Demo and replace; or supply and install any missing, damaged or painted receptacle, wall plates and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace; or supply and install any missing, damaged interior light globes or trim.

- Install new LED bulbs throughout the interior.

- Demo and replace all carpet flooring with spec sheet vinyl. To include supply and install spec underlayment. Vinyl selection to be 58021 Sunstone.

- Demo and replace toe kick on kitchen cabinets. Match existing finish.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Thoroughly clean all window surfaces.

- Supply and install any missing; or demo and replace any damaged knobs or hardware for both bath vanities. Thoroughly clean cabinet, sink, faucet and countertop.

- Demo and replace both showerheads with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor in each bathroom.

- Thoroughly clean tub and surrounds.

- Contractor shall complete repair of upper bathroom toilet. Contractor may use parts onsite. Contractor shall supply and install any parts needed to repair the toilet in the upper bathroom.

- Contractor shall demo and replace the showerhead and trim set for the upper bathroom per Spec Book.

- Thoroughly clean toilet stools.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Supply and install any missing door-stops throughout with spec product.

- Adjust all interior doors.

- Service existing furnace and HRV unit.

- Supply and install new hot water heater per Spec Book.

- Supply and install standing laundry tub per Spec Book.

- Thoroughly clean all interior surfaces.

2485 Evergreen Ct Unit D

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Demo and replace garage service door lock jamb, weatherstripping and hardware. Clean and painting door slab for reuse. Finish per Spec Book. Install new lock set with dead bolt to be keyed alike front and rear entries.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing  as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint Dover White. Demo and replace sheetrock for areas with extensive mold.

- Demo and replace; or supply and install any missing, damaged or painted receptacle, wall plates and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace all interior light fixtures per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace all flooring with spec sheet vinyl. Vinyl selection to be 58020 Oasis Beige.

- Demo and replace existing range hood with Whirlpool model UXT4030ADW.

- Demo and replace all lower kitchen cabinets and countertop to match existing finishes. This should include removal of the existing dishwasher and installing a 24” cabinet in its place. Reinstall existing sink and faucet.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Thoroughly clean all window surfaces.

- Supply and install new window blinds throughout.

- Supply and install any missing; or demo and replace any damaged knobs or hardware for bath vanity. Thoroughly clean cabinet, sink, faucet and countertop.

- Demo and replace showerhead with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor in each bathroom.

- Thoroughly clean tub and surround.

- Contractor shall thoroughly clean existing toilet.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Supply and install any missing door-stops throughout with spec product.

- Adjust all interior doors.

- Service existing furnace and HRV unit.

- Thoroughly clean all interior surfaces.

2489 Evergreen Ct Quad Exterior: The following allows to the entire exterior of the quad. Work specific to the exterior of each unit is listed separately.

Units A, B, C & D:

- Demo and replace all damaged siding and accessories. All replaced siding shall be color matched to existing. This shall include any siding with excessive screw holes.

- Pressure wash and clean all siding and exterior surfaces.

- Demo and replace; or supply and install any damaged or missing foundation flashing.

- Contractor shall demo and dispose of patio fence.

- Demo and replace exterior light fixtures with Spec model.

- Supply and install all new LED bulbs for exterior light fixtures.

- Demo and replace damaged or missing downspouts; and secure all.

- Cleanout all gutters.

- Supply and install spec splash pads.

- All entry door lock sets to be keyed alike, including front, patio and garage service.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing or demo and replace damaged window or patio door screens.

- Contractor shall remove and dispose of the existing dish installed on the roof. Contractor shall caulk screw holes or repair any damage to the roof.

- Contractor shall supply and install any missing outlet box covers.

- Contractor shall remove all incidental small trees and vegetation growing next to the exterior of the unit.

- Contractor shall remove and dispose of any miscellaneous yard debris.

2489 Evergreen Ct Unit A

Exterior:

- Contractor shall demo and replace “sunken” section of sidewalk. Contractor shall supply and install any clean fill necessary to prep the area. The new section shall have a slight incline to match the elevation of the existing sidewalk on either side.

- Demo and replace overhead garage door, weatherstripping and metal jambs.

- Contractor shall adjust and paint front entry door Autumn Red. Finish per Spec Book.

2489 Evergreen Ct Unit B

Exterior:

- Contractor shall demo and replace doorbell.

- Contractor shall demo and replace storm door, including chain kit.

- Contractor shall demo and replace front entry door slab, lock jamb and weatherstripping. New door shall be painted Autumn Red. Finish per Spec Book.

2489 Evergreen Ct Unit C

Exterior:

- Contractor shall demo and replace doorbell.

- Contractor shall adjust and paint front entry door Autumn Red. Finish per Spec Book.

- Supply and install new lock sets for both the front entry and patio doors to be keyed alike and the same as garage service door.

2489 Evergreen Ct Unit D

Exterior:

- Contractor shall demo and replace cracked section of sidewalk. Contractor shall supply and install any clean fill necessary to prep the area. The new section shall have a slight incline to match the elevation of the existing sidewalk on either side.

- Demo and replace overhead garage door weatherstripping and metal jambs.

- Demo and replace damaged; or supply and install missing metal foundation flashing.

- Contractor shall demo and replace doorbell.

- Contractor shall demo and replace storm door with Spec model including chain kit.

- Contractor shall demo and replace front entry door slab, lock jamb and weatherstripping. Paint new door Autumn Red. Finish per Spec Book.

2489 Evergreen Ct Unit A

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Adjust garage service door. Demo and replace weatherstripping. Install new lock set with deadbolt to be keyed alike front and rear entries.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing  as closely as possible. Repaint Dover White.

- Demo and replace; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace damaged; or supply and install any missing light fixtures or lighting components per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace existing range hood with Whirlpool model UXT4030ADW.

- Contractor shall remove and dispose of existing dishwasher. Supply and install 24” cabinet in its place. Match to existing finishes.

- Demo and replace toe kick on kitchen cabinets. Match existing finish.

- Contractor shall cleanup kitchen sink caulk and/ or re-caulk.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install liner for cabinet beneath kitchen sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Thoroughly clean all window surfaces.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Supply and install any missing; or demo and replace any damaged knobs or hardware for bath vanity. Thoroughly clean cabinet, sink, faucet and countertop.

- Demo and replace both showerhead with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor.

- Thoroughly clean tub and surround.

- Contractor shall demo and replace toilet stool.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Supply and install any missing door-stops throughout with spec product.

- Contractor shall demo and replace bathroom door, casing and hardware. Match existing finishes.

- Adjust all interior doors.

- Contractor shall reassemble the HRV unit. Service existing furnace and HRV unit.

- Thoroughly clean all interior surfaces.

2489 Evergreen Ct Unit B

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Demo and replace garage service door slab, weatherstripping and lock jamb. Install new lock set with dead bolt to be keyed alike front and rear entries.

- Contractor shall patch and tape damaged area of firewall in the garage.

- Contractor shall have the low voltage box inspected for damage and make any necessary repairs. Contractor shall reinstall.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing as closely as possible. Repaint Dover White.

- Demo and replace; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Contractor shall demo and replace all vinyl stair treads.

- Contractor shall demo and replace damaged flooring section of dining/ living room on the lower level and the upper rear bedroom floor in vinyl flooring per Spec Book. Vinyl selection to be 58021 Sunstone.

- Demo and replace damaged; or supply and install any missing light fixtures or lighting components per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace existing range hood with Whirlpool model UXT4030ADW.

- Demo and replace all lower kitchen cabinets and countertop to match existing finishes. This should include removal of the existing dishwasher and installing a 24” cabinet in its place. Thoroughly clean and reinstall existing sink and faucet.

- Thoroughly clean all kitchen cabinet surfaces. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Thoroughly clean all window surfaces.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Supply and install any missing; or demo and replace any damaged knobs or hardware for either bath vanity. Thoroughly clean cabinets, sinks, faucets and countertops.

- Demo and replace both showerhead with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor.

- Thoroughly clean tub and surround.

- Contractor shall demo and replace both toilet stools.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Supply and install any missing door-stops throughout with spec product.

- Contractor shall demo and replace damaged bedroom and lower bath doors; and supply and install upper bathroom door.

- Contractor shall reinstall bedroom closet sliding doors.

- Adjust all interior doors.

- Contractor shall reassemble the HRV unit. Service existing furnace and HRV unit.

- Thoroughly clean all interior surfaces.

2489 Evergreen Ct Unit C

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Adjust and paint garage service door. Install new lock set with dead bolt to be keyed alike front and rear entries.

- Contractor shall have the low voltage box inspected for damage and make any necessary repairs. Contractor shall supply and install a new cover.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing  as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint Dover White. Demo and replace sheetrock for areas with extensive mold.

- Demo and replace; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Contractor shall demo and replace all damaged or missing vinyl stair treads.

- Contractor shall demo and replace flooring in the upper rear bedroom floor in vinyl flooring per Spec Book. Vinyl selection to be 58021 Sunstone.

- Demo and replace damaged; or supply and install any missing light fixtures or lighting components per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace all lower kitchen cabinets to match existing finishes. Thoroughly clean and reinstall existing countertop and sink.

- Contractor shall demo and replace kitchen faucet.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Thoroughly clean all window surfaces.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Demo and replace bath vanity cabinets. Thoroughly clean sinks, faucets and countertops.

- Demo and replace both showerhead with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor.

- Thoroughly clean tub and surround.

- Contractor shall thoroughly clean both toilet stools.

- Re-caulk counter tops, toilet stool base, tub/shower surrounds, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Supply and install any missing door-stops throughout with spec product.

- Contractor shall demo and replace damaged door to upper rear bedroom.

- Adjust all interior doors.

- Contractor shall reinstall sliding doors for upper rear bedroom closets.

- Service existing furnace and HRV unit.

- Thoroughly clean all interior surfaces.

- Contractor shall remove and dispose of any miscellaneous debris left in the unit.

2489 Evergreen Ct Unit D

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Clean and adjust garage service door. Install new lock set with deadbolt to be keyed alike front and rear entries.

- Contractor shall inspect low voltage box and complete any necessary repairs. Contractor shall supply and install cover.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all interior drywall surfaces. Texturing to match existing  as closely as possible. Repaint Dover White.

- Demo and replace; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo and replace; or supply and install all new floor vents.

- Demo and replace damaged; or supply and install any missing light fixtures or lighting components per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace existing range hood with Whirlpool model UXT4030ADW.

- Contractor shall remove and dispose of existing dishwasher. Supply and install 24” cabinet in its place. Match to existing finishes.

- Demo and replace all lower kitchen cabinet drawer and door fronts and toe kick. Match existing finish.

- Thoroughly clean all kitchen cabinet surfaces, to include counter tops and kitchen sink. Supply and install any missing cabinet drawers, shelving, doors cutting board, hardware, knobs. Any new hardware and stain should match existing finishes. Adjust all cabinets.

- Demo and replace damaged; or supply and install any missing baseboards, trim or casing. Match to existing finishes.

- Contractor shall demo and replace all flooring. Vinyl selection shall be 58021 Sunstone. Clean and paint mechanical room floor per Spec Book.

- Thoroughly clean all window surfaces.

- Contractor shall supply and install missing; or demo and replace damaged wire shelving per Spec Book.

- Contractor shall demo and replace toe kick and broken door on the bath vanity. Match existing finish.

- Supply and install any missing; or demo and replace any damaged knobs or hardware for bath vanity. Thoroughly clean cabinet, sink, faucet and countertop.

- Demo and replace showerhead with Moen product similar to Spec model.

- Demo and replace Broan bath fan grille and fan assembly to include motor.

- Thoroughly clean tub and surround.

- Contractor shall thoroughly clean toilet stool.

- Re-caulk counter tops, toilet stool base, tub/shower surround, as needed.

- Supply and install missing; or demo and replace damaged bath accessories.

- Supply and install any missing door-stops throughout with spec product.

- Contractor shall demo and replace 2 interior doors for bedroom and bath.

- Adjust all interior doors.

- Contractor shall demo any existing and supply and install all new blinds, including window and patio door.

- Service existing furnace and HRV unit.

- Contractor shall supply and install mechanical room floor drain cover.

- Thoroughly clean all interior surfaces.

Detached Garage –

- Demo and replace all damaged siding and accessories. All replaced siding shall be color matched to existing. Contractor shall supply and install Certainteed or L.P. Smartside Woodgrain one piece corners per Spec Book.

- Pressure wash and clean all siding and exterior surfaces.

- Contractor shall demo and replace overhead garage door, weatherstripping, metal jambs and frame.

- Contractor shall repair soffit by entry door.

- Contractor shall demo and replace the entry door and weatherstripping.

- Supply and install new lockset with deadbolt.

- Contractor shall demo and replace exterior lighting per Spec Book and install new LED bulbs.

Fence – Contractor shall supply lump sum cost to provide materials and labor for installation of a chain link fence as follows in the base bid.

- Contractor shall supply and install black vinyl coated chain link fence around the perimeter of the 3 quad units as shown on the provided layout. Approximate fence perimeter including gates totals 1,380 ft. Contractor shall include 1,500 ft of fence in their base bid. Unit cost per ft to install shall be provided in Bid Alternate #1 and will be used to calculate add or deduct based on actual install. See following product specifications.

- Contractor shall supply and install (2) cantilever slide gates 30’ in width, one left & one right slide at driveway entrances.

- Contractor shall remove all trees, brush, overhanging branches and shrubs necessary to install fence.

- Contractor shall include the cost of any digging or other land alterations required to install fence.

- Owner shall apply for and obtain conditional use permit through the City of Hinckley.

SECTION 02830 - FENCES AND GATES

PART 1 GENERAL

1.01 SUMMARY

A. Section Includes:

1. Chain link fence.

2. Gates.

1.02 REFERENCES

A. ASTM A53, Specification for Pipe, Steel, Black and Hot-Dipped, Zinc-Coated Welded and

Seamless.

B. ASTM A392, Specification for Zinc-Coated Steel Chain-Link Fence Fabric.

C. Chain Link Fence Manufacturers Institute (CLFMI) Product Manual.

PART 2 PRODUCTS

2.01 FENCE POSTS

A. The pipe shall be round steel pipe with galvanized coating, Allied Tube SS30, in the

following diameters:

1. Posts

a. Fence height: 6’

b. Corner Posts: 3” O.D.

c. Top, Intermediate and Bottom Rails: 1 5/8” O.D.

2. Intermediate or Diagonal Bracing: 1 5/8” O.D.

3. Gate Posts: refer to Manufacturers recommendations.

4. Gate Frames: refer to Manufacturers recommendations.

B. The above pipe diameters are the minimum allowable. The actual pipe diameters and

weights of the fence posts and rails shall be determined by the installer based on local

building codes, fence heights, wind speed design requirements and coverage of the fence

fabric with windscreens, plywood or other wind blocking materials.

SECTION 02830 – FENCES & GATES

Page 2

2.02 FENCE FABRIC

A. For Standard Fences: Zinc-coated (ASTM 392) steel wire fabric, 9 gauge, 2-inch square

with a minimum coating of 1.2 oz per sq. ft. of surface area. The top and bottom selvages

shall be knuckled, unless otherwise specified.

B. Fasteners.

1. To terminal posts:

a. Tension bars: Minimum 1/4 inch by 3/4 inch.

b. Clamps: 12 gauge by 1 inch wide.

c. Carriage bolts: Minimum 5/16-inch diameter.

2. To line posts: Wire: 6-gauge aluminum.

3. To rails: Wire: 6-gauge aluminum.

C. Tension and Brace Bands.

1. The tension and brace bands shall be 0.078” by 3/4” for posts less than 4” O.D. with

5/16” diameter galvanized carriage bolts.

2. The tension and brace bands shall be 0.108” by 7/8” for posts 4” O.D. and larger

with 5/16” diameter galvanized carriage bolts.

D. Tension Bars.

1. The tension bars shall be a minimum of 3/16” by 3/4” galvanized steel or vinyl

coated where used with vinyl coated fences.

E. Tie Wire.

1. The tie wire shall be 6 gauge aluminum wire.

F. Accessories.

1. Provide galvanized steel or malleable iron post caps, sleeves, clamps,

hinges, gate latches, keepers, and all other accessories required

for the complete installation of the chain link fence and gates. Provide vinyl coated

accessories for all vinyl coated fences.

2.03 RAILS

A. Top and bottom rails shall be installed on all standard fences. Install center rails on fences 6’

and higher.

B. Top, center, and bottom rails shall be installed on all tennis court fences.

C. Install rails for relocated backstops to original configuration.

2.04 DOME CAPS

A. Galvanized malleable steel dome caps shall be provided at all posts.

2.05 GATE ACCESSORIES

A. Gate hinges shall be galvanized steel of adequate strength to support the gate with large

bearing surfaces for clamping the gate in position. The hinges shall not twist or turn under

the action of the gate under any conditions of use or abuse.

B. Locking devices shall be galvanized steel with a central locking device for double gates and a

forked latch for single gates.

C. Disability gate latches shall be galvanized steel with an extended lever to lift the forked latch

on single gate.

2.06 BRACING

A. The fence bracing shall be 1-5/8 inch outside diameter pipe as required.

PART 3 EXECUTION

3.01 INSTALLATION

A. Remove and dispose of brush, trees, and other obstructions that interfere with construction of

the fence.

B. Lay out and measure the fence line parallel to the contour of the ground.

C. Post Setting.

1. Set standard corner, end, intermediate pull, and gate posts into concrete footings with

a minimum diameter of 12 inches and a minimum depth of 54 inches. Place the

posts such that bottom of the footing is 6 inches deeper than the bottom of the post.

Place the concrete in a continuous pour and tamp around the posts for consolidation.

Line posts may be air driven to a minimum depth of 4 feet. The top of the posts shall

not be deformed at completion. Replace all deformed or damaged posts.

2. Crown the tops of the footings to shed water.

3. Space standard fence posts a maximum of 10 feet on-center.

D. Posts and Rails.

1. Set the terminal posts at the beginning and the end of each continuous, straight-line

length of fence and at abrupt changes in vertical or horizontal alignment.

2. Set line posts at intervals stated above, measured parallel to the grade of the fence.

3. Install intermediate pull posts at a maximum interval of 200 feet.

4. Install a continuous top rail. Install the top rail through loop caps on the line posts.

Provide sleeve splices in the top rail and only cut the pipe at the ends of runs. Install

a bottom rail with terminations at each post. Install middle rails as specified.

5. Provide horizontal bracing at corner posts, terminal posts, and intermediate pull

posts. Install the bracing at the mid-height of the fence, between the corner,

terminal, or pull posts and the next adjacent line post. Fasten the bracing to the posts

with brace bands and carriage bolts.

E. Fabric.

1. Stretch the fabric and tension wire, if applicable, to the proper tension between the

terminal posts as recommended by the manufacturer. Fasten the fabric securely to

the framework.

2. Attach the fabric to the terminal posts with tension bar clamps and carriage bolts.

3. Attach the fabric to the line posts at intervals of 14” on center, and to the top, middle

and bottom rails at intervals of 14” on center for all standard fences.

4. Install the fabric a maximum of 1” above the finished elevations.

F. Gates.

1. Provide manually operated, cantilever slide gates of the sizes indicated. Furnish all required

hardware, latches, and locking devices.

2. Assemble the gate frames with welds or with specially fabricated fittings. Paint

welds with aluminum based paint upon completion.

3. Attach the gate fabric to the gate frame at 14” intervals on all sides. Use the same

fabric as the adjacent fence.

4. Provide latches, plunger bar if required, and padlock hasps at all gates.

G. Adjustments.

1. Hardware and Accessories.

a. Adjust all hardware, including cantilever gate tracks and rollers, latches, and

locking devices for the proper operation without sticking, binding, or dragging.

Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact either Carla Dunkley at 320-630-2495 or by email at carla.dunkley@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact TERO Office at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: Evergreen Ct Quad Rehabs

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

- Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.  Site will also require City of Hinckley permitting.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2019 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  Evergreen Ct Quad Rehabs

LUMP SUM PRICE: 2481 Evergreen Ct

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE: 2485 Evergreen Ct

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE: 2489 Evergreen Ct

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE: Detached Garage

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE: Fence

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE FOR ALL:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1: Unit Cost Per Ft of Fence Installed

$_________/ ft

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)                       °    LETTER FROM BONDING SURETY

- MLBO VENDOR LICENSE                                        °    SUB-CONTRACTOR LISTS (Include values)

- COPY OF CURRENT INSURANCES                        °    COPY OF MINNESOTA CONTRACTOR’S LICENSE

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________