Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:   September 9, 2019

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          September 25, 2019

PROJECT:      46367 Alma Razor Rd Insulation & Heating

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a residential renovation at 46367 Alma Razor Rd, Hinckley MN.   Bids will be due Wednesday September 25, 2019 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday, September 26, 2019 at 8:30 AM.

All interested bidders must notify me by September 17, 2019.

On September 18, 2019, I will make arrangements to schedule a mandatory pre-bid site visit for the interested bidders, who did not attend the previous pre-bid site visit, to attend. Depending on scheduling of pre-bid site visit, the bid deadline may be extended. I will issue a pre-bid addendum as soon as possible to all interested bidders informing them of the pre-bid site visit and deadline extension, if either are determined necessary; or to confirm that no pre-bid site visit will be required and to confirm that the deadline will remain the same.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:

Bid Option #1:

- Remove and dispose of all existing attic insulation throughout the house.

- Remove any existing poly vapor barrier above ceiling.

- Enclose opening around chimney pipe with sheetrock.

- Ensure that chimney pipe meets all code required clearances within attic space.

- Examine all heating supply vents to ensure that they are properly connected.

- Demo and replace and existing flex duct with hard duct.

- Contractor shall ensure that the home’s ductwork includes adequate supply and return air runs. Contractor shall supply and install any additional venting and ductwork required.

- Demo any existing attic chutes without insulation stops. Replace those removed with new venting with insulation stops at exterior walls.

- Sprayfoam ductwork to current code standards.

- Sprayfoam attic floor to depth of 2” throughout.

- Install 6” of blown fiberglass insulation throughout. Spray foam and blown fiberglass insulation layers shall meet R38 value.

- Remove and dispose of existing furnace.

- Contractor shall supply and install new Lennox furnace with a-coil and HRV per Spec Book. Contractor shall ensure that models are properly sized for the home.

- Contractor shall demo existing thermostat and replace with Honeywell T6 Pro Series TH6210U2001.

- Contractor shall flush the existing AC unit then properly reconnect and recharge.

- Demo and replace all ceiling sheetrock in front bedroom.

- Paint and finish bedroom ceiling to match existing color but without texturing.

- Demo existing and install 2 new properly insulated attic accesses.

- For all exterior wall cavities found to be uninsulated below the kitchen/ dining room window, drill a hole large enough to allow for insulation to be blown into cavity to meet current code of R21 value.

- Patch, texture and paint wall openings to match existing drywall kitchen finish.

- Sand and repaint water damage in kitchen and living room ceiling caused from previous condensation. Match existing color and finish.

Bid Option #2:

- Remove and dispose of all existing attic insulation throughout the house.

- Remove any existing poly vapor barrier above ceiling.

- Enclose opening around chimney pipe with sheetrock.

- Ensure that chimney pipe meets all code required clearances within attic space.

- Examine all heating supply vents to ensure that they are properly connected.

- Contractor shall ensure that the home’s existing flex duct includes adequate supply and return air runs. Contractor shall supply and install any additional venting and flex duct required.

- Contractor shall supply and install two (2) 6” Vents – US In-Line duct fans with built-in silencers, model: TTSILENTM150. One fan shall be installed in each of the supply ducts feeding to the bedrooms.

- Demo any existing attic chutes without insulation stops. Replace those removed with new venting with insulation stops at exterior walls.

- Sprayfoam attic floor to depth of 2” throughout.

- Install 6” of blown fiberglass insulation throughout. Spray foam and blown fiberglass insulation layers shall meet R38 value.

- Remove and dispose of existing furnace.

- Contractor shall supply and install new Lennox furnace with a-coil and HRV per Spec Book. Contractor shall ensure that models are properly sized for the home.

- Contractor shall demo existing thermostat and replace with Honeywell T6 Pro Series TH6210U2001.

- Contractor shall flush the existing AC unit then properly reconnect and recharge.

- Demo and replace all ceiling sheetrock in front bedroom.

- Paint and finish bedroom ceiling to match existing color but without texturing.

- Demo existing and install 2 new properly insulated attic accesses.

- For all exterior wall cavities found to be uninsulated below the kitchen/ dining room window, drill a hole large enough to allow for insulation to be blown into cavity to meet current code of R21 value.

- Patch, texture and paint wall openings to match existing drywall kitchen finish.

- Sand and repaint water damage in kitchen and living room ceiling caused from previous condensation. Match existing color and finish.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Carla Dunkley – CMD Project Coordinator at carla.dunkley@millelacsband.com or at 320.532.7429 to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 46367 Alma Razor Rd Insulation & Heating

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2019 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  46367 Alma Razor Rd Hinckley, MN Insulation & Heating

BID OPTION #1 LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

BID OPTION #2 LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________