Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:   30 July 2018

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:         15 August 2018

PROJECT:      5418 Cty. Rd. 138, Garrison

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for renovation work at 5418 Cty. Rd. 138, Garrison, MN.   Bids will be due Wednesday 15 August, 2018 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday 16 August, 2018 at 8:30 AM.

A mandatory Pre-Bid site visit will be held at the site on Monday August 6th  at 3:00 PM.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:

Interior;

- Demo and replace main level flooring to include vinyl plank flooring (TrafficMaster Allure Plank White Maple Model 97011) in bedrooms, bath, hall, laundry, kitchen. Include new underlayment installation, and subfloor repair in the 2nd bedroom.  Install base shoe moulding where vinyl flooring is installed.

- Demo and replace living room flooring with new carpet (TrafficMaster Soared Sky High sku 304698223).

- Demo and replace bathtub shower unit with remodeler shower unit.  Warm Rain, sized to fit existing location H-533FF.  Provide and install new shower door assembly.  Include $600 material cost allowance in the base bid.  Provide homeowner with selection options compatible with the new shower assembly. Install new shower valve and head.  Install drywall around shower enclosure, tape, finish and paint.

- Demo and replace bathroom vanity.  Include new cultured marble top with integral sink and new faucet.  Install ¼” drywall behind vanity wall, tape, finish and paint.

- Demo and replace bathroom exhaust fan.  Vent to exterior and re-wire, utilizing motion sensor plug and play module for fan control.  Set occupant timer for 20min.

- Repair electrical circuit to living room.

- Demo and replace electric water heater with 40gal AO Smith propane unit.  Measure and ensure new heater will fit the existing bedroom enclosure.  Complete propane supply and vent hook up per code.

- Demo and replace water softener unit. When removed, clean and remove salt deposits from the surrounding walls and millwork.

- Demo and replace kitchen cabinetry (Stain: Varathane Briarsmoke), countertops (Wilsonart HD Sumer Carnival 1875K-35 Mirage Finish), sink and faucet.  Provide and install new dishwasher unit to the left of the sink.  Demo closet next to entry door, and extend base and upper cabinets through.  Utilize paneling removed for patching exterior wall.  Install ¼” drywall behind cabinet walls, tape and finish.  Paint finish hto be semi-gloss.  Homeowner to select color.  Demo base and upper cabinetry from short wall.  Install new upper cabinets in this location.  Patch fastener holes where base cabinets are removed.  Run the new flooring through where base cabinets were previous.  Re-install appliances.  See Plan layout.

- Clean, and re-paint existing ceilings.  Repair ceiling materials, patch to match existing texture. Prime with Kilz stain blocking primer.

- Replace bathroom and bedroom door slabs.  Match to existing, and re-install door hardware.

Exterior;

- Replace or Re-glaze broken bedroom window sash.

- Demo and replace main entry door.  Install ½ lite door.  Install new entry hardware set, keyed alike.

- Demo and replace front entry deck and stair assembly 10’x12’.  Relocate new stair to the front corner of the deck facing the road.  Install sonotube footings and column base connections to the new pier supports.  Install 6’x6’ concrete pad at the base of the stairs.  Include new handrail and perimeter guardrail sections as required by code.

- Provide Storage Pod for homeowner.  Deliver 2 weeks prior to renovation start and leave on site 2 weeks after completion.

- Provide 2 yard dumpster for homeowner use.  Deliver 2 weeks prior to renovation start.

## Bid Alternate #1: None

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Ryan Jendro – CMD Project Manager at ryan.jendro@millelacsband.com or at 320.630.5011 to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - A copy of Subcontractor/Material Supplier list

  - A copy of valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Craig Hansen at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: (5418 Cty 138)

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

- Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@mlcv.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permit & Crow Wing.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2018 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  5418 Cty. Rd. 138, Garrison

LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1: (Description)

__________________________________________________________________$____________________

(Written Value)                                                                                                 (Dollar Amount)

ALTERNATE #2 (Description):

__________________________________________________________________$____________________

(Written Value)                                                                                                 (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________